Multiplexer 16:1 logic

MUX16to1.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity MUX16to1 is

port (

D : in std_logic_vector(15 downto 0);

S : in std_logic_vector(3 downto 0);

Y : out std_logic

);

end MUX16to1;

architecture GateLevel of MUX16to1 is

signal W : std_logic_vector(15 downto 0);

begin

W(0) <= D(0) and (not S(3)) and (not S(2)) and (not S(1)) and (not S(0));

W(1) <= D(1) and (not S(3)) and (not S(2)) and (not S(1)) and S(0);

W(2) <= D(2) and (not S(3)) and (not S(2)) and S(1) and (not S(0));

W(3) <= D(3) and (not S(3)) and (not S(2)) and S(1) and S(0);

W(4) <= D(4) and (not S(3)) and S(2) and (not S(1)) and (not S(0));

W(5) <= D(5) and (not S(3)) and S(2) and (not S(1)) and S(0);

W(6) <= D(6) and (not S(3)) and S(2) and S(1) and (not S(0));

W(7) <= D(7) and (not S(3)) and S(2) and S(1) and S(0);

W(8)  <= D(8) and S(3) and (not S(2)) and (not S(1)) and (not S(0));

W(9)  <= D(9) and S(3) and (not S(2)) and (not S(1)) and S(0);

W(10) <= D(10) and S(3) and (not S(2)) and S(1) and (not S(0));

W(11) <= D(11) and S(3) and (not S(2)) and S(1) and S(0);

W(12) <= D(12) and S(3) and S(2) and (not S(1)) and (not S(0));

W(13) <= D(13) and S(3) and S(2) and (not S(1)) and S(0);

W(14) <= D(14) and S(3) and S(2) and S(1) and (not S(0));

W(15) <= D(15) and S(3) and S(2) and S(1) and S(0);

Y <= W(0) or W(1) or W(2) or W(3) or

W(4) or W(5) or W(6) or W(7) or

W(8) or W(9) or W(10) or W(11) or

W(12) or W(13) or W(14) or W(15);

end GateLevel;

tb_MUX16to1.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity tb_MUX16to1 is

end tb_MUX16to1;

architecture sim of tb_MUX16to1 is

signal D0  : std_logic := '0';

signal D1  : std_logic := '0';

signal D2  : std_logic := '0';

signal D3  : std_logic := '0';

signal D4  : std_logic := '0';

signal D5  : std_logic := '0';

signal D6  : std_logic := '0';

signal D7  : std_logic := '0';

signal D8  : std_logic := '0';

signal D9  : std_logic := '0';

signal D10 : std_logic := '0';

signal D11 : std_logic := '0';

signal D12 : std_logic := '0';

signal D13 : std_logic := '0';

signal D14 : std_logic := '0';

signal D15 : std_logic := '0';

signal D : std_logic_vector(15 downto 0);

signal S : std_logic_vector(3 downto 0);

signal Y : std_logic;

begin

D <= D15 & D14 & D13 & D12 &

D11 & D10 & D9 & D8 &

D7 & D6 & D5 & D4 &

D3 & D2 & D1 & D0;

uut : entity work.MUX16to1

port map

(

D => D,

S => S,

Y => Y

);

process

begin

D0 <= '1';

S  <= "0000";

wait for 10 ns;

D0 <= '0';

D1 <= '1';

S  <= "0001";

wait for 10 ns;

D1 <= '0';

D2 <= '1';

S  <= "0010";

wait for 10 ns;

D2 <= '0';

D3 <= '1';

S  <= "0011";

wait for 10 ns;

D3 <= '0';

D4 <= '1';

S  <= "0100";

wait for 10 ns;

D4 <= '0';

D5 <= '1';

S  <= "0101";

wait for 10 ns;

D5 <= '0';

D6 <= '1';

S  <= "0110";

wait for 10 ns;

D6 <= '0';

D7 <= '1';

S  <= "0111";

wait for 10 ns;

D7 <= '0';

D8 <= '1';

S  <= "1000";

wait for 10 ns;

D8 <= '0';

D9 <= '1';

S  <= "1001";

wait for 10 ns;

D9 <= '0';

D10 <= '1';

S   <= "1010";

wait for 10 ns;

D10 <= '0';

D11 <= '1';

S   <= "1011";

wait for 10 ns;

D11 <= '0';

D12 <= '1';

S   <= "1100";

wait for 10 ns;

D12 <= '0';

D13 <= '1';

S   <= "1101";

wait for 10 ns;

D13 <= '0';

D14 <= '1';

S   <= "1110";

wait for 10 ns;

D14 <= '0';

D15 <= '1';

S   <= "1111";

wait for 10 ns;

D15 <= '0';

wait;

end process;

end sim;

MUX16-1-wave.vcd

Demultiplexer 1:16 logic

DMUX1to16.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity DMUX1to16 is

port (

D : in std_logic;

S : in std_logic_vector(3 downto 0);

Y : out std_logic_vector(15 downto 0)

);

end DMUX1to16;

architecture GateLevel of DMUX1to16 is

begin

Y(0) <= D and (not S(3)) and (not S(2)) and (not S(1)) and (not S(0)); -- 0000

Y(1) <= D and (not S(3)) and (not S(2)) and (not S(1)) and S(0);

Y(2) <= D and (not S(3)) and (not S(2)) and S(1) and (not S(0));

Y(3) <= D and (not S(3)) and (not S(2)) and S(1) and S(0);

Y(4) <= D and (not S(3)) and S(2) and (not S(1)) and (not S(0));

Y(5) <= D and (not S(3)) and S(2) and (not S(1)) and S(0);

Y(6) <= D and (not S(3)) and S(2) and S(1) and (not S(0));

Y(7) <= D and (not S(3)) and S(2) and S(1) and S(0);

Y(8)  <= D and S(3) and (not S(2)) and (not S(1)) and (not S(0));

Y(9)  <= D and S(3) and (not S(2)) and (not S(1)) and S(0);

Y(10) <= D and S(3) and (not S(2)) and S(1) and (not S(0));

Y(11) <= D and S(3) and (not S(2)) and S(1) and S(0);

Y(12) <= D and S(3) and S(2) and (not S(1)) and (not S(0));

Y(13) <= D and S(3) and S(2) and (not S(1)) and S(0);

Y(14) <= D and S(3) and S(2) and S(1) and (not S(0));

Y(15) <= D and S(3) and S(2) and S(1) and S(0); -- 1111

end GateLevel;

tb_DMUX1to16.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity tb_DMUX1to16 is

end tb_DMUX1to16;

architecture sim of tb_DMUX1to16 is

signal D : std_logic := '0';

signal S : std_logic_vector(3 downto 0);

signal Y : std_logic_vector(15 downto 0);

signal Y0  : std_logic;

signal Y1  : std_logic;

signal Y2  : std_logic;

signal Y3  : std_logic;

signal Y4  : std_logic;

signal Y5  : std_logic;

signal Y6  : std_logic;

signal Y7  : std_logic;

signal Y8  : std_logic;

signal Y9  : std_logic;

signal Y10 : std_logic;

signal Y11 : std_logic;

signal Y12 : std_logic;

signal Y13 : std_logic;

signal Y14 : std_logic;

signal Y15 : std_logic;

begin

uut : entity work.DMUX1to16

port map

(

D => D,

S => S,

Y => Y

);

Y0  <= Y(0);

Y1  <= Y(1);

Y2  <= Y(2);

Y3  <= Y(3);

Y4  <= Y(4);

Y5  <= Y(5);

Y6  <= Y(6);

Y7  <= Y(7);

Y8  <= Y(8);

Y9  <= Y(9);

Y10 <= Y(10);

Y11 <= Y(11);

Y12 <= Y(12);

Y13 <= Y(13);

Y14 <= Y(14);

Y15 <= Y(15);

process

begin

D <= '1';

S <= "0000";

wait for 10 ns;

S <= "0001";

wait for 10 ns;

S <= "0010";

wait for 10 ns;

S <= "0011";

wait for 10 ns;

S <= "0100";

wait for 10 ns;

S <= "0101";

wait for 10 ns;

S <= "0110";

wait for 10 ns;

S <= "0111";

wait for 10 ns;

S <= "1000";

wait for 10 ns;

S <= "1001";

wait for 10 ns;

S <= "1010";

wait for 10 ns;

S <= "1011";

wait for 10 ns;

S <= "1100";

wait for 10 ns;

S <= "1101";

wait for 10 ns;

S <= "1110";

wait for 10 ns;

S <= "1111";

wait for 10 ns;

D <= '0';

wait;

end process;

end sim;

DMUX1-16-wave.vcd