ใบงานที่ 5

เรื่อง การทดลองวงจร BCD Arithmetic Unit

จัดทำโดย

นาย ศิริพงศ์ บุญประสิทธิ์

รหัส ENGCE200_SEC1

เสนอ

อาจารย์สมนึก สุระธง

ใบงานนี้เป็นส่วนหนึ่งของวิชา

ENGCE200

การออกแบบดิจิทัล

(Digital Systems Design)

หลักสูตร วศ.บ.วิศวกรรมคอมพิวเตอร์

สาขาวิศวกรรมไฟฟ้า คณะวิศวกรรมศาสตร์

มหาวิทยาลัยเทคโนโลยีราชมงคลล้านนา ภาคพายัพ เชียงใหม่

ภาคเรียนที่ 1 ปีการศึกษา 2569

ใบงานที่ 5

ชื่อบทเรียน  การทดลองวงจร BCD Arithmetic Unit

จุดประสงค์การสอน  ปฏิบัติการทดลองวงจร BCD Arithmetic Unit

ออกแบบวงจร BCD Arithmetic Unit

ทดสอบวงจร BCD Arithmetic Unit

1. ทฤษฎี

การออกแบบวงจร BCD Arithmetic Unit

การออกแบบวงจร BCD Adder/Sub Tractor ฐานสิบ 3 หลัก

2. จงออกแบบวงจรต่อไปนี้

แสดงขั้นตอนการออกแบบวงจร BCD Adder/Sub ฐานสิบ 3 หลัก คือ หลักหน่วย , หลักสิบและหลักร้อย สามารถใช้ MSI IC ได้

3. ผลการทดลอง

4. สรุปผลการทดลอง

5. คำถามหลังการทดลอง

- บอกหลักการแปลง binary adder มาสร้างเป็น BCD adder

- บอกหลักการทำวงจรBCDบวกให้เป็นวงจรลบBCD

- ถ้าต้องการออกแบบวงจรBCDบวกและลบเลขขนาด 5 หลัก จะต้องออกแบบอย่างไร

Design.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity BCD_Adder_Subtractor is

Port (

A_H, A_T, A_U : in  STD_LOGIC_VECTOR(3 downto 0);

B_H, B_T, B_U : in  STD_LOGIC_VECTOR(3 downto 0);

M             : in  STD_LOGIC;

-- M=0 ADD

-- M=1 SUB

S_H, S_T, S_U : out STD_LOGIC_VECTOR(3 downto 0);

COUT          : out STD_LOGIC

);

end BCD_Adder_Subtractor;

architecture Behavioral of BCD_Adder_Subtractor is

begin

process(A_H,A_T,A_U,B_H,B_T,B_U,M)

variable a_num  : integer;

variable b_num  : integer;

variable result : integer;

variable h,t,u  : integer;

begin

a_num :=

to_integer(unsigned(A_H))*100 +

to_integer(unsigned(A_T))*10 +

to_integer(unsigned(A_U));

b_num :=

to_integer(unsigned(B_H))*100 +

to_integer(unsigned(B_T))*10 +

to_integer(unsigned(B_U));

if (M='0') then

result := a_num + b_num;

else

result := a_num - b_num;

end if;

if result < 0 then

result := 0;

end if;

h := result / 100;

t := (result mod 100) / 10;

u := result mod 10;

S_H <= std_logic_vector(to_unsigned(h,4));

S_T <= std_logic_vector(to_unsigned(t,4));

S_U <= std_logic_vector(to_unsigned(u,4));

if(result > 999) then

COUT <= '1';

else

COUT <= '0';

end if;

end process;

end Behavioral;

Tb.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_BCD_Adder_Subtractor is

end tb_BCD_Adder_Subtractor;

architecture Behavioral of tb_BCD_Adder_Subtractor is

component BCD_Adder_Subtractor

Port (

A_H, A_T, A_U : in  STD_LOGIC_VECTOR(3 downto 0);

B_H, B_T, B_U : in  STD_LOGIC_VECTOR(3 downto 0);

M             : in  STD_LOGIC;

S_H, S_T, S_U : out STD_LOGIC_VECTOR(3 downto 0);

COUT          : out STD_LOGIC

);

end component;

signal A_H,A_T,A_U : STD_LOGIC_VECTOR(3 downto 0);

signal B_H,B_T,B_U : STD_LOGIC_VECTOR(3 downto 0);

signal S_H,S_T,S_U : STD_LOGIC_VECTOR(3 downto 0);

signal M           : STD_LOGIC;

signal COUT        : STD_LOGIC;

begin

UUT: BCD_Adder_Subtractor

port map(

A_H => A_H,

A_T => A_T,

A_U => A_U,

B_H => B_H,

B_T => B_T,

B_U => B_U,

M => M,

S_H => S_H,

S_T => S_T,

S_U => S_U,

COUT => COUT

);

stim_proc : process

begin

------------------------------------------------

-- TEST 1 : 245 + 378 = 623

------------------------------------------------

M <= '0';

A_H <= "0010";

A_T <= "0100";

A_U <= "0101";

B_H <= "0011";

B_T <= "0111";

B_U <= "1000";

wait for 100 ns;

------------------------------------------------

-- TEST 2 : 589 + 478 = 1067

------------------------------------------------

M <= '0';

A_H <= "0101";

A_T <= "1000";

A_U <= "1001";

B_H <= "0100";

B_T <= "0111";

B_U <= "1000";

wait for 100 ns;

------------------------------------------------

-- TEST 3 : 635 - 214 = 421

------------------------------------------------

M <= '1';

A_H <= "0110";

A_T <= "0011";

A_U <= "0101";

B_H <= "0010";

B_T <= "0001";

B_U <= "0100";

wait for 100 ns;

------------------------------------------------

-- TEST 4 : 500 - 125 = 375

------------------------------------------------

M <= '1';

A_H <= "0101";

A_T <= "0000";

A_U <= "0000";

B_H <= "0001";

B_T <= "0010";

B_U <= "0101";

wait for 100 ns;

wait;

end process;

end Behavioral;