ใบงานที่ 1

เรื่อง การทดลองวงจรมัลติเพลกเซอร์และวงจรดีมัลติเพลกเซอร์

(Multiplexer and Demultiplexer)

จัดทำโดย

นาย ศิริพงศ์ บุญประสิทธิ์

รหัส ENGCE200_SEC1

เสนอ

อาจารย์สมนึก สุระธง

ใบงานนี้เป็นส่วนหนึ่งของวิชา

ENGCE200

การออกแบบดิจิทัล

(Digital Systems Design)

หลักสูตร วศ.บ.วิศวกรรมคอมพิวเตอร์

สาขาวิศวกรรมไฟฟ้า คณะวิศวกรรมศาสตร์

มหาวิทยาลัยเทคโนโลยีราชมงคลล้านนา ภาคพายัพ เชียงใหม่

ภาคเรียนที่ 1 ปีการศึกษา 2568

ใบงานที่ 1

ชื่อบทเรียน  การทดลองวงจรมัลติเพลกเซอร์และวงจรดีมัลติเพลกเซอร์

(Multiplexer and Demultiplexer)

จุดประสงค์การสอน

ปฏิบัติการทดลองสร้างวงจรคอมไบเนชันด้วยลอจิกเกตพื้นฐาน

ออกแบบและทดสอบวงจรมัลติเพล็กซ์

ออกแบบและทดสอบวงจรดีมัลติเพล็กซ์

1. ทฤษฎี

วิธีการออกแบบวงจรมัลติเพล็กซ์ (Multiplexer)

วิธีการออกแบบวงจรดีมัลติเพล็กซ์ (Demultiplexer)

2. จงออกแบบวงจรต่อไปนี้

1. วงจรมัลติเพล็กซ์ (16 : 1 Multiplexer)

2. วงจรดีมัลติเพล็กซ์ (1 : 16 Demultiplexer)

3. ผลการทดลอง

ทดลองโดยการนำ Output Multiplexer ต่อ เข้า Input Demultiplexer แล้วเลือก select อิสระ

4. สรุปผลการทดลอง

5. คำถามหลังการทดลอง

1. บอกหลักการทำงานวงจรมัลติเพล็กซ์

2. บอกหลักการทำงานวงจรดีมัลติเพล็กซ์

3. บอกวิธีการเชื่อมต่อทางไกลโดยสายส่งคู่เดียว

Multiplexer 16:1 logic

MUX16to1.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity MUX16to1 is

port (

D : in std_logic_vector(15 downto 0);

S : in std_logic_vector(3 downto 0);

Y : out std_logic

);

end MUX16to1;

architecture GateLevel of MUX16to1 is

signal W : std_logic_vector(15 downto 0);

begin

W(0) <= D(0) and (not S(3)) and (not S(2)) and (not S(1)) and (not S(0));

W(1) <= D(1) and (not S(3)) and (not S(2)) and (not S(1)) and S(0);

W(2) <= D(2) and (not S(3)) and (not S(2)) and S(1) and (not S(0));

W(3) <= D(3) and (not S(3)) and (not S(2)) and S(1) and S(0);

W(4) <= D(4) and (not S(3)) and S(2) and (not S(1)) and (not S(0));

W(5) <= D(5) and (not S(3)) and S(2) and (not S(1)) and S(0);

W(6) <= D(6) and (not S(3)) and S(2) and S(1) and (not S(0));

W(7) <= D(7) and (not S(3)) and S(2) and S(1) and S(0);

W(8)  <= D(8) and S(3) and (not S(2)) and (not S(1)) and (not S(0));

W(9)  <= D(9) and S(3) and (not S(2)) and (not S(1)) and S(0);

W(10) <= D(10) and S(3) and (not S(2)) and S(1) and (not S(0));

W(11) <= D(11) and S(3) and (not S(2)) and S(1) and S(0);

W(12) <= D(12) and S(3) and S(2) and (not S(1)) and (not S(0));

W(13) <= D(13) and S(3) and S(2) and (not S(1)) and S(0);

W(14) <= D(14) and S(3) and S(2) and S(1) and (not S(0));

W(15) <= D(15) and S(3) and S(2) and S(1) and S(0);

Y <= W(0) or W(1) or W(2) or W(3) or

W(4) or W(5) or W(6) or W(7) or

W(8) or W(9) or W(10) or W(11) or

W(12) or W(13) or W(14) or W(15);

end GateLevel;

tb_MUX16to1.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity tb_MUX16to1 is

end tb_MUX16to1;

architecture sim of tb_MUX16to1 is

signal D0  : std_logic := '0';

signal D1  : std_logic := '0';

signal D2  : std_logic := '0';

signal D3  : std_logic := '0';

signal D4  : std_logic := '0';

signal D5  : std_logic := '0';

signal D6  : std_logic := '0';

signal D7  : std_logic := '0';

signal D8  : std_logic := '0';

signal D9  : std_logic := '0';

signal D10 : std_logic := '0';

signal D11 : std_logic := '0';

signal D12 : std_logic := '0';

signal D13 : std_logic := '0';

signal D14 : std_logic := '0';

signal D15 : std_logic := '0';

signal D : std_logic_vector(15 downto 0);

signal S : std_logic_vector(3 downto 0);

signal Y : std_logic;

begin

D <= D15 & D14 & D13 & D12 &

D11 & D10 & D9 & D8 &

D7 & D6 & D5 & D4 &

D3 & D2 & D1 & D0;

uut : entity work.MUX16to1

port map

(

D => D,

S => S,

Y => Y

);

process

begin

D0 <= '1';

S  <= "0000";

wait for 10 ns;

D0 <= '0';

D1 <= '1';

S  <= "0001";

wait for 10 ns;

D1 <= '0';

D2 <= '1';

S  <= "0010";

wait for 10 ns;

D2 <= '0';

D3 <= '1';

S  <= "0011";

wait for 10 ns;

D3 <= '0';

D4 <= '1';

S  <= "0100";

wait for 10 ns;

D4 <= '0';

D5 <= '1';

S  <= "0101";

wait for 10 ns;

D5 <= '0';

D6 <= '1';

S  <= "0110";

wait for 10 ns;

D6 <= '0';

D7 <= '1';

S  <= "0111";

wait for 10 ns;

D7 <= '0';

D8 <= '1';

S  <= "1000";

wait for 10 ns;

D8 <= '0';

D9 <= '1';

S  <= "1001";

wait for 10 ns;

D9 <= '0';

D10 <= '1';

S   <= "1010";

wait for 10 ns;

D10 <= '0';

D11 <= '1';

S   <= "1011";

wait for 10 ns;

D11 <= '0';

D12 <= '1';

S   <= "1100";

wait for 10 ns;

D12 <= '0';

D13 <= '1';

S   <= "1101";

wait for 10 ns;

D13 <= '0';

D14 <= '1';

S   <= "1110";

wait for 10 ns;

D14 <= '0';

D15 <= '1';

S   <= "1111";

wait for 10 ns;

D15 <= '0';

wait;

end process;

end sim;

MUX16-1-wave.vcd

Demultiplexer 1:16 logic

DMUX1to16.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity DMUX1to16 is

port (

D : in std_logic;

S : in std_logic_vector(3 downto 0);

Y : out std_logic_vector(15 downto 0)

);

end DMUX1to16;

architecture GateLevel of DMUX1to16 is

begin

Y(0) <= D and (not S(3)) and (not S(2)) and (not S(1)) and (not S(0)); -- 0000

Y(1) <= D and (not S(3)) and (not S(2)) and (not S(1)) and S(0);

Y(2) <= D and (not S(3)) and (not S(2)) and S(1) and (not S(0));

Y(3) <= D and (not S(3)) and (not S(2)) and S(1) and S(0);

Y(4) <= D and (not S(3)) and S(2) and (not S(1)) and (not S(0));

Y(5) <= D and (not S(3)) and S(2) and (not S(1)) and S(0);

Y(6) <= D and (not S(3)) and S(2) and S(1) and (not S(0));

Y(7) <= D and (not S(3)) and S(2) and S(1) and S(0);

Y(8)  <= D and S(3) and (not S(2)) and (not S(1)) and (not S(0));

Y(9)  <= D and S(3) and (not S(2)) and (not S(1)) and S(0);

Y(10) <= D and S(3) and (not S(2)) and S(1) and (not S(0));

Y(11) <= D and S(3) and (not S(2)) and S(1) and S(0);

Y(12) <= D and S(3) and S(2) and (not S(1)) and (not S(0));

Y(13) <= D and S(3) and S(2) and (not S(1)) and S(0);

Y(14) <= D and S(3) and S(2) and S(1) and (not S(0));

Y(15) <= D and S(3) and S(2) and S(1) and S(0); -- 1111

end GateLevel;

tb_DMUX1to16.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity tb_DMUX1to16 is

end tb_DMUX1to16;

architecture sim of tb_DMUX1to16 is

signal D : std_logic := '0';

signal S : std_logic_vector(3 downto 0);

signal Y : std_logic_vector(15 downto 0);

signal Y0  : std_logic;

signal Y1  : std_logic;

signal Y2  : std_logic;

signal Y3  : std_logic;

signal Y4  : std_logic;

signal Y5  : std_logic;

signal Y6  : std_logic;

signal Y7  : std_logic;

signal Y8  : std_logic;

signal Y9  : std_logic;

signal Y10 : std_logic;

signal Y11 : std_logic;

signal Y12 : std_logic;

signal Y13 : std_logic;

signal Y14 : std_logic;

signal Y15 : std_logic;

begin

uut : entity work.DMUX1to16

port map

(

D => D,

S => S,

Y => Y

);

Y0  <= Y(0);

Y1  <= Y(1);

Y2  <= Y(2);

Y3  <= Y(3);

Y4  <= Y(4);

Y5  <= Y(5);

Y6  <= Y(6);

Y7  <= Y(7);

Y8  <= Y(8);

Y9  <= Y(9);

Y10 <= Y(10);

Y11 <= Y(11);

Y12 <= Y(12);

Y13 <= Y(13);

Y14 <= Y(14);

Y15 <= Y(15);

process

begin

D <= '1';

S <= "0000";

wait for 10 ns;

S <= "0001";

wait for 10 ns;

S <= "0010";

wait for 10 ns;

S <= "0011";

wait for 10 ns;

S <= "0100";

wait for 10 ns;

S <= "0101";

wait for 10 ns;

S <= "0110";

wait for 10 ns;

S <= "0111";

wait for 10 ns;

S <= "1000";

wait for 10 ns;

S <= "1001";

wait for 10 ns;

S <= "1010";

wait for 10 ns;

S <= "1011";

wait for 10 ns;

S <= "1100";

wait for 10 ns;

S <= "1101";

wait for 10 ns;

S <= "1110";

wait for 10 ns;

S <= "1111";

wait for 10 ns;

D <= '0';

wait;

end process;

end sim;

DMUX1-16-wave.vcd

mux_dmux16 curcuit

mux_dmux16.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity mux_dmux16 is

port (

D : in std_logic_vector(15 downto 0);

S_MUX  : in std_logic_vector(3 downto 0);

S_DMUX : in std_logic_vector(3 downto 0);

Y : out std_logic_vector(15 downto 0)

);

end mux_dmux16;

architecture Structural of mux_dmux16 is

signal M : std_logic;

begin

----------------------------------------------------------------

-- MUX 16:1

----------------------------------------------------------------

M <= (D(0) and not S_MUX(3) and not S_MUX(2) and not S_MUX(1) and not S_MUX(0))

or (D(1) and not S_MUX(3) and not S_MUX(2) and not S_MUX(1) and S_MUX(0))

or (D(2) and not S_MUX(3) and not S_MUX(2) and S_MUX(1) and not S_MUX(0))

or (D(3) and not S_MUX(3) and not S_MUX(2) and S_MUX(1) and S_MUX(0))

or (D(4) and not S_MUX(3) and S_MUX(2) and not S_MUX(1) and not S_MUX(0))

or (D(5) and not S_MUX(3) and S_MUX(2) and not S_MUX(1) and S_MUX(0))

or (D(6) and not S_MUX(3) and S_MUX(2) and S_MUX(1) and not S_MUX(0))

or (D(7) and not S_MUX(3) and S_MUX(2) and S_MUX(1) and S_MUX(0))

or (D(8) and S_MUX(3) and not S_MUX(2) and not S_MUX(1) and not S_MUX(0))

or (D(9) and S_MUX(3) and not S_MUX(2) and not S_MUX(1) and S_MUX(0))

or (D(10) and S_MUX(3) and not S_MUX(2) and S_MUX(1) and not S_MUX(0))

or (D(11) and S_MUX(3) and not S_MUX(2) and S_MUX(1) and S_MUX(0))

or (D(12) and S_MUX(3) and S_MUX(2) and not S_MUX(1) and not S_MUX(0))

or (D(13) and S_MUX(3) and S_MUX(2) and not S_MUX(1) and S_MUX(0))

or (D(14) and S_MUX(3) and S_MUX(2) and S_MUX(1) and not S_MUX(0))

or (D(15) and S_MUX(3) and S_MUX(2) and S_MUX(1) and S_MUX(0));

----------------------------------------------------------------

-- DMUX 1:16

----------------------------------------------------------------

Y(0) <= M and not S_DMUX(3) and not S_DMUX(2) and not S_DMUX(1) and not S_DMUX(0);

Y(1) <= M and not S_DMUX(3) and not S_DMUX(2) and not S_DMUX(1) and S_DMUX(0);

Y(2) <= M and not S_DMUX(3) and not S_DMUX(2) and S_DMUX(1) and not S_DMUX(0);

Y(3) <= M and not S_DMUX(3) and not S_DMUX(2) and S_DMUX(1) and S_DMUX(0);

Y(4) <= M and not S_DMUX(3) and S_DMUX(2) and not S_DMUX(1) and not S_DMUX(0);

Y(5) <= M and not S_DMUX(3) and S_DMUX(2) and not S_DMUX(1) and S_DMUX(0);

Y(6) <= M and not S_DMUX(3) and S_DMUX(2) and S_DMUX(1) and not S_DMUX(0);

Y(7) <= M and not S_DMUX(3) and S_DMUX(2) and S_DMUX(1) and S_DMUX(0);

Y(8)  <= M and S_DMUX(3) and not S_DMUX(2) and not S_DMUX(1) and not S_DMUX(0);

Y(9)  <= M and S_DMUX(3) and not S_DMUX(2) and not S_DMUX(1) and S_DMUX(0);

Y(10) <= M and S_DMUX(3) and not S_DMUX(2) and S_DMUX(1) and not S_DMUX(0);

Y(11) <= M and S_DMUX(3) and not S_DMUX(2) and S_DMUX(1) and S_DMUX(0);

Y(12) <= M and S_DMUX(3) and S_DMUX(2) and not S_DMUX(1) and not S_DMUX(0);

Y(13) <= M and S_DMUX(3) and S_DMUX(2) and not S_DMUX(1) and S_DMUX(0);

Y(14) <= M and S_DMUX(3) and S_DMUX(2) and S_DMUX(1) and not S_DMUX(0);

Y(15) <= M and S_DMUX(3) and S_DMUX(2) and S_DMUX(1) and S_DMUX(0);

end Structural;

tb_mux_dmux16.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity tb_mux_dmux16 is

end tb_mux_dmux16;

architecture sim of tb_mux_dmux16 is

signal D : std_logic_vector(15 downto 0);

signal S_MUX  : std_logic_vector(3 downto 0);

signal S_DMUX : std_logic_vector(3 downto 0);

signal Y : std_logic_vector(15 downto 0);

-- แตก D ออกเป็นรายเส้น

signal D0  : std_logic:= '0';

signal D1  : std_logic:= '0';

signal D2  : std_logic:= '0';

signal D3  : std_logic:= '0';

signal D4  : std_logic:= '0';

signal D5  : std_logic:= '0';

signal D6  : std_logic:= '0';

signal D7  : std_logic:= '0';

signal D8  : std_logic:= '0';

signal D9  : std_logic:= '0';

signal D10 : std_logic:= '0';

signal D11 : std_logic:= '0';

signal D12 : std_logic:= '0';

signal D13 : std_logic:= '0';

signal D14 : std_logic:= '0';

signal D15 : std_logic:= '0';

-- แตก Y ออกเป็นรายเส้น

signal Y0  : std_logic;

signal Y1  : std_logic;

signal Y2  : std_logic;

signal Y3  : std_logic;

signal Y4  : std_logic;

signal Y5  : std_logic;

signal Y6  : std_logic;

signal Y7  : std_logic;

signal Y8  : std_logic;

signal Y9  : std_logic;

signal Y10 : std_logic;

signal Y11 : std_logic;

signal Y12 : std_logic;

signal Y13 : std_logic;

signal Y14 : std_logic;

signal Y15 : std_logic;

begin

----------------------------------------------------------------

-- รวม D0-D15 เป็น Bus

----------------------------------------------------------------

D <= D15 & D14 & D13 & D12 &

D11 & D10 & D9 & D8 &

D7 & D6 & D5 & D4 &

D3 & D2 & D1 & D0;

----------------------------------------------------------------

-- แยก Y

----------------------------------------------------------------

Y0  <= Y(0);

Y1  <= Y(1);

Y2  <= Y(2);

Y3  <= Y(3);

Y4  <= Y(4);

Y5  <= Y(5);

Y6  <= Y(6);

Y7  <= Y(7);

Y8  <= Y(8);

Y9  <= Y(9);

Y10 <= Y(10);

Y11 <= Y(11);

Y12 <= Y(12);

Y13 <= Y(13);

Y14 <= Y(14);

Y15 <= Y(15);

----------------------------------------------------------------

-- DUT

----------------------------------------------------------------

uut : entity work.mux_dmux16

port map

(

D      => D,

S_MUX  => S_MUX,

S_DMUX => S_DMUX,

Y      => Y

);

----------------------------------------------------------------

-- Truth Table Test

----------------------------------------------------------------

process

begin

D0     <= '1';

S_MUX  <= "0000";

S_DMUX <= "0000";

wait for 10 ns;

D0 <= '0';

D1     <= '1';

S_MUX  <= "0001";

S_DMUX <= "0001";

wait for 10 ns;

D1 <= '0';

D2     <= '1';

S_MUX  <= "0010";

S_DMUX <= "0010";

wait for 10 ns;

D2 <= '0';

D3     <= '1';

S_MUX  <= "0011";

S_DMUX <= "0011";

wait for 10 ns;

D3 <= '0';

D4     <= '1';

S_MUX  <= "0100";

S_DMUX <= "0100";

wait for 10 ns;

D4 <= '0';

D5     <= '1';

S_MUX  <= "0101";

S_DMUX <= "0101";

wait for 10 ns;

D5 <= '0';

D6     <= '1';

S_MUX  <= "0110";

S_DMUX <= "0110";

wait for 10 ns;

D6 <= '0';

D7     <= '1';

S_MUX  <= "0111";

S_DMUX <= "0111";

wait for 10 ns;

D7 <= '0';

D8     <= '1';

S_MUX  <= "1000";

S_DMUX <= "1000";

wait for 10 ns;

D8 <= '0';

D9     <= '1';

S_MUX  <= "1001";

S_DMUX <= "1001";

wait for 10 ns;

D9 <= '0';

D10    <= '1';

S_MUX  <= "1010";

S_DMUX <= "1010";

wait for 10 ns;

D10 <= '0';

D11    <= '1';

S_MUX  <= "1011";

S_DMUX <= "1011";

wait for 10 ns;

D11 <= '0';

D12    <= '1';

S_MUX  <= "1100";

S_DMUX <= "1100";

wait for 10 ns;

D12 <= '0';

D13    <= '1';

S_MUX  <= "1101";

S_DMUX <= "1101";

wait for 10 ns;

D13 <= '0';

D14    <= '1';

S_MUX  <= "1110";

S_DMUX <= "1110";

wait for 10 ns;

D14 <= '0';

D15    <= '1';

S_MUX  <= "1111";

S_DMUX <= "1111";

wait for 10 ns;

D15 <= '0';

wait;

end process;

end sim;