ใบงานที่ 3

เรื่อง การทดลองใช้ MSI ออกแบบวงจรเปรียบเทียบ

จัดทำโดย

นาย ศิริพงศ์ บุญประสิทธิ์

รหัส ENGCE200_SEC1

เสนอ

อาจารย์สมนึก สุระธง

ใบงานนี้เป็นส่วนหนึ่งของวิชา

ENGCE200

การออกแบบดิจิทัล

(Digital Systems Design)

หลักสูตร วศ.บ.วิศวกรรมคอมพิวเตอร์

สาขาวิศวกรรมไฟฟ้า คณะวิศวกรรมศาสตร์

มหาวิทยาลัยเทคโนโลยีราชมงคลล้านนา ภาคพายัพ เชียงใหม่

ภาคเรียนที่ 1 ปีการศึกษา 2569

ใบงานที่ 3

ชื่อบทเรียน  การทดลองใช้ MSI ออกแบบวงจรเปรียบเทียบ

จุดประสงค์การสอน ปฏิบัติการทดลองใช้ MSI ออกแบบวงจรเปรียบเทียบ

ออกแบบและทดสอบวงจรเปรียบเทียบ โดยใช้ MSI

1. ทฤษฎี

วิธีออกแบบวงจรด้วย Variable-Entering Map

วิธีการออกแบบวงจร โดยใช้ 4 : 1 Multiplex

#### 2. จงออกแบบวงจรเปรียบเทียบ 4 บิท กำหนดให้

#### 2.1 มี Input เป็น A  มี 4 บิท และ B มี 4 บิท  สามารถเปรียบเทียบ น้อยกว่า,มากกว่า, เท่ากับ

#### 2.2 ใช้ 4 : 1 Multiplex

3. ผลการทดลอง

4. สรุปผลการทดลอง

5. คำถามหลังการทดลอง

- บอกข้อดีและข้อเสียการใช้ MSI ออกแบบวงจร

- บอกหลักการใช้ Multiplex ออกแบบวงจร

- เมื่อไรถึงจะใช้หลักการ Variable-Entering Map

| A | B | A2 | A1 | A0 | B2 | B1 | B0 | A>B | A<B | A=B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| 0 | 2 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 0 | 3 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 0 | 4 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |
| 0 | 5 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 0 | 6 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 0 | 7 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 1 | 2 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 3 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 4 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 5 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 6 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 7 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 2 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 2 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 2 | 2 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 2 | 3 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 2 | 4 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |
| 2 | 5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 2 | 6 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 2 | 7 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
| 3 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 3 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 3 | 2 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |
| 3 | 3 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| 3 | 4 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 3 | 5 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 3 | 6 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 3 | 7 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 4 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 4 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 4 | 2 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 4 | 3 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |
| 4 | 4 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 4 | 5 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 4 | 6 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 4 | 7 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
| 5 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 5 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 5 | 2 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |
| 5 | 3 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| 5 | 4 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 5 | 5 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| 5 | 6 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 5 | 7 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 6 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 6 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 6 | 2 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 6 | 3 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |
| 6 | 4 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 6 | 5 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 6 | 6 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 6 | 7 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
| 7 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 7 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 7 | 2 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |
| 7 | 3 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| 7 | 4 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 7 | 5 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 |
| 7 | 6 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 7 | 7 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |

Test.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity test is

Port (

A2, A1, A0 : in STD_LOGIC;

B2, B1, B0 : in STD_LOGIC;

A_more_B   : out STD_LOGIC;

A_less_B   : out STD_LOGIC;

A_equal_B  : out STD_LOGIC

);

end test;

architecture Behavioral of test is

begin

process(A2, A1, A0, B2, B1, B0)

variable SEL : STD_LOGIC_VECTOR(1 downto 0);

begin

SEL := A2 & A1;

case SEL is

when "00" => A_more_B <= (A0 and not B2 and not B1 and not B0);

when "01" => A_more_B <= ((not B2 and not B1) or (A0 and not B2 and B1 and not B0));

when "10" => A_more_B <= (B2 or (A0 and B2 and not B1 and not B0));

when others => A_more_B <= (not B2 or not B1 or (A0 and B2 and B1 and not B0));

end case;

end process;

process(A2, A1, A0, B2, B1, B0)

variable SEL : STD_LOGIC_VECTOR(1 downto 0);

begin

SEL := A2 & A1;

case SEL is

when "00" => A_less_B <= (B1 or B2 or (not A0 and not B2 and not B1 and B0));

when "01" => A_less_B <= (B2 or (not A0 and not B2 and B1 and B0));

when "10" => A_less_B <= ((B2 and B1) or (not A0 and B2 and not B1 and B0));

when others => A_less_B <= (not A0 and B2 and B1 and B0);

end case;

end process;

process(A2, A1, A0, B2, B1, B0)

variable SEL : STD_LOGIC_VECTOR(1 downto 0);

begin

SEL := A2 & A1;

case SEL is

when "00" => A_equal_B <= ((not A0 and not B2 and not B1 and not B0) or (A0 and not B2 and not B1 and B0));

when "01" => A_equal_B <= ((A0 and not B2 and B1 and B0) or (not A0 and not B2 and B1 and not B0));

when "10" => A_equal_B <= ((not A0 and B2 and not B1 and not B0) or (A0 and B2 and not B1 and B0));

when others => A_equal_B <= ((A0 and B2 and B1 and B0) or (not A0 and B2 and B1 and not B0));

end case;

end process;

end Behavioral;

TB_Test.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_test is

end tb_test;

architecture Behavioral of tb_test is

component test

Port (

A2, A1, A0 : in STD_LOGIC;

B2, B1, B0 : in STD_LOGIC;

A_more_B   : out STD_LOGIC;

A_less_B   : out STD_LOGIC;

A_equal_B  : out STD_LOGIC

);

end component;

signal A2, A1, A0 : STD_LOGIC := '0';

signal B2, B1, B0 : STD_LOGIC := '0';

signal A_more_B, A_less_B, A_equal_B : STD_LOGIC;

begin

UUT: test

port map (

A2 => A2,

A1 => A1,

A0 => A0,

B2 => B2,

B1 => B1,

B0 => B0,

A_more_B => A_more_B,

A_less_B => A_less_B,

A_equal_B => A_equal_B

);

stimulus : process

begin

-- A=2, B=2  => A=B

A2<='0'; A1<='1'; A0<='0';

B2<='0'; B1<='1'; B0<='0';

wait for 10 ns;

-- A=3, B=1  => A>B

A2<='0'; A1<='1'; A0<='1';

B2<='0'; B1<='0'; B0<='1';

wait for 10 ns;

-- A=4, B=5  => A<B

A2<='1'; A1<='0'; A0<='0';

B2<='1'; B1<='0'; B0<='1';

wait for 10 ns;

wait;

end process;

end Behavioral;

Old_Test.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity test is

Port (

A2, A1, A0 : in STD_LOGIC;

B2, B1, B0 : in STD_LOGIC;

A_more_B   : out STD_LOGIC;

A_less_B   : out STD_LOGIC;

A_equal_B  : out STD_LOGIC

);

end test;

architecture Behavioral of test is

begin

-- A>B

A_more_B  <= (not A2 and not A1) and (A0 and not B2 and not B1 and not B0)or (not A2 and A1) and((not B2 and not B1) or(A0 and not B2 and B1 and not B0))or (A2 and not A1) and(B2 or(A0 and B2 and not B1 and not B0))or (A2 and A1) and(not B2 or not B1 or(A0 and B2 and B1 and not B0));

-- A<B

A_less_B  <= (not A2 and not A1) and(B1 or B2 or(not A0 and not B2 and not B1 and B0)) or (not A2 and A1) and(B2 or(not A0 and not B2 and B1)) or (A2 and not A1) and((B2 and B1) or(not A0 and B2 and not B1 and B0)) or (A2 and A1) and(not A0 and B2 and B1 and B0);

-- A=B

A_equal_B <= (not A2 and not A1) and((not A0 and not B2 and not B1 and not B0) or(A0 and not B2 and not B1 and B0)) or (not A2 and A1) and((A0 and not B2 and B1 and B0) or(not A0 and not B2 and B1 and not B0)) or (A2 and not A1) and((not A0 and B2 and not B1 and not B0) or(A0 and B2 and not B1 and B0)) or (A2 and A1) and((A0 and B2 and B1 and B0) or(not A0 and B2 and B1 and not B0));

end Behavioral;