ใบงานที่ 2

เรื่อง ทดลองวงจรถอดรหัสและวงจรเข้ารหัส

(Encoder and Decoder)

จัดทำโดย

นาย ศิริพงศ์ บุญประสิทธิ์

รหัส ENGCE200_SEC1

เสนอ

อาจารย์สมนึก สุระธง

ใบงานนี้เป็นส่วนหนึ่งของวิชา

ENGCE200

การออกแบบดิจิทัล

(Digital Systems Design)

หลักสูตร วศ.บ.วิศวกรรมคอมพิวเตอร์

สาขาวิศวกรรมไฟฟ้า คณะวิศวกรรมศาสตร์

มหาวิทยาลัยเทคโนโลยีราชมงคลล้านนา ภาคพายัพ เชียงใหม่

ภาคเรียนที่ 1 ปีการศึกษา 2568

ใบงานที่ 2

ชื่อบทเรียน  ทดลองวงจรถอดรหัสและวงจรเข้ารหัส

(Encoder and Decoder)

จุดประสงค์การสอน

ปฏิบัติการทดลองวงจรถอดรหัสและวงจรเข้ารหัส (Encoder and Decoder)

ออกแบบและทดสอบวงจรเข้ารหัส

ออกแบบและทดสอบวงจร์ถอดรหัส

1. ทฤษฎี

วิธีการออกแบบวงจรเข้ารหัส

วิธีการออกแบบวงจรดีถอดรหัส

2. ให้ออกแบบวงจรถอดรหัสและวงจรเข้ารหัส (Encoder and Decoder) ดังรูปที่ 1

รูปที่ 1

ให้ทำวงจรสแกน Kaypad ให้สามารถทงานได้โดยการ กดค้างไวก็แสดงเลขนั้นที่ 7-segment

3. ผลการทดลอง

4. สรุปผลการทดลอง

5. คำถามหลังการทดลอง

1. บอกหลักการทำงานวงจรเข้ารหัส

2. บอกหลักการทำงานวงจรถอดรหัส

3. ถ้าจะต่อให้แสดงตัวเลขเป็นสองหลักต้องทำอย่างไร

encoder-decoder-7segment.circ

encoder-decoder-7segment.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity test is

Port (

D0 : in STD_LOGIC;

D1 : in STD_LOGIC;

D2 : in STD_LOGIC;

D3 : in STD_LOGIC;

D4 : in STD_LOGIC;

D5 : in STD_LOGIC;

D6 : in STD_LOGIC;

D7 : in STD_LOGIC;

D8 : in STD_LOGIC;

D9 : in STD_LOGIC;

seg_a : out STD_LOGIC;

seg_b : out STD_LOGIC;

seg_c : out STD_LOGIC;

seg_d : out STD_LOGIC;

seg_e : out STD_LOGIC;

seg_f : out STD_LOGIC;

seg_g : out STD_LOGIC

);

end test;

architecture Behavioral of test is

signal A,B,C,D : std_logic;

signal na,nb,nc,nd : std_logic;

begin

--------------------------------------------------

-- Encoder

--------------------------------------------------

A <= D8 or D9;

B <= D4 or D5 or D6 or D7;

C <= D2 or D3 or D6 or D7;

D <= D1 or D3 or D5 or D7 or D9;

--------------------------------------------------

-- NOT Gate

--------------------------------------------------

NA <= not A;

NB <= not B;

NC <= not C;

ND <= not D;

--------------------------------------------------

-- Decoder

--------------------------------------------------

seg_a <= A or C or (B and D) or (NB and ND);

seg_b <= NB or (C and D) or (NC and ND);

seg_c <= B or NC or D;

seg_d <= A or (NB and C) or (NB and ND) or (C and ND) or (B and NC and D);

seg_e <= (NB and ND) or (C and ND);

seg_f <= A or (B and NC) or (C and ND) or (NC and ND);

seg_g <= A or (NB and C) or (B and NC) or (B and ND);

end Behavioral;

tb_encoder-decoder-7segment.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_test is

end tb_test;

architecture Behavioral of tb_test is

component test

Port (

D0 : in STD_LOGIC;

D1 : in STD_LOGIC;

D2 : in STD_LOGIC;

D3 : in STD_LOGIC;

D4 : in STD_LOGIC;

D5 : in STD_LOGIC;

D6 : in STD_LOGIC;

D7 : in STD_LOGIC;

D8 : in STD_LOGIC;

D9 : in STD_LOGIC;

seg_a : out STD_LOGIC;

seg_b : out STD_LOGIC;

seg_c : out STD_LOGIC;

seg_d : out STD_LOGIC;

seg_e : out STD_LOGIC;

seg_f : out STD_LOGIC;

seg_g : out STD_LOGIC

);

end component;

signal D0,D1,D2,D3,D4,D5,D6,D7,D8,D9 : STD_LOGIC := '0';

signal seg_a,seg_b,seg_c,

seg_d,seg_e,seg_f,seg_g : STD_LOGIC;

begin

UUT : test

port map(

D0=>D0, D1=>D1, D2=>D2, D3=>D3, D4=>D4,

D5=>D5, D6=>D6, D7=>D7, D8=>D8, D9=>D9,

seg_a=>seg_a,

seg_b=>seg_b,

seg_c=>seg_c,

seg_d=>seg_d,

seg_e=>seg_e,

seg_f=>seg_f,

seg_g=>seg_g

);

stim_proc : process

begin

D0<='1'; wait for 20 ns;

D0<='0';

D1<='1'; wait for 20 ns;

D1<='0';

D2<='1'; wait for 20 ns;

D2<='0';

D3<='1'; wait for 20 ns;

D3<='0';

D4<='1'; wait for 20 ns;

D4<='0';

D5<='1'; wait for 20 ns;

D5<='0';

D6<='1'; wait for 20 ns;

D6<='0';

D7<='1'; wait for 20 ns;

D7<='0';

D8<='1'; wait for 20 ns;

D8<='0';

D9<='1'; wait for 20 ns;

D9<='0';

wait;

end process;

end Behavioral;