ใบงานที่ 4

เรื่อง การทดลองวงจรหน่วยประมวลผลคณิตศาสตร์และตรรกวิทยา

จัดทำโดย

นาย ศิริพงศ์ บุญประสิทธิ์

รหัส ENGCE200_SEC1

เสนอ

อาจารย์สมนึก สุระธง

ใบงานนี้เป็นส่วนหนึ่งของวิชา

ENGCE200

การออกแบบดิจิทัล

(Digital Systems Design)

หลักสูตร วศ.บ.วิศวกรรมคอมพิวเตอร์

สาขาวิศวกรรมไฟฟ้า คณะวิศวกรรมศาสตร์

มหาวิทยาลัยเทคโนโลยีราชมงคลล้านนา ภาคพายัพ เชียงใหม่

ภาคเรียนที่ 1 ปีการศึกษา 2569

ใบงานที่ 4

ชื่อบทเรียน  การทดลองวงจรหน่วยประมวลผลคณิตศาสตร์และตรรกวิทยา

จุดประสงค์การสอน  ปฏิบัติการทดลองวงจรหน่วยประมวลผลคณิตศาสตร์และตรรกวิทยา

ออกแบบวงจรหน่วยประมวลผลคณิตศาสตร์และตรรกวิทยา

1. ทฤษฎี

วิธีการออกแบบวงจรบวก

วิธีการออกแบบวงจรลบ

วิธีการออกแบบวงจรตรรกวิทยา

2. จงออกแบบวงจรวงจร 5  Bit ALU

รูปที่ 1  บล็อกไดอะแกรม 5 Bit ALU

รูปที่ 2 บล็อกไดอะแกรมภายใน 5 Bit ALU มีสองส่วน

ตารางที่ 1: Functions of ALU

| M | S1 | S0 | C0 | FUNCTION | OPERATION |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | A | Transfer A |
| 0 | 0 | 0 | 1 | A + 1 | Increment A by 1 |
| 0 | 0 | 1 | 0 | A + B | Add A and B |
| 0 | 0 | 1 | 1 | A + B + 1 | Increment the sum of A and B by 1 |
| 0 | 1 | 0 | 0 | A + B' | A plus one's complement of B |
| 0 | 1 | 0 | 1 | A - B | Subtract B from A (i.e. B' + A + 1) |
| 0 | 1 | 1 | 0 | A' + B | B plus one's complement of A |
| 0 | 1 | 1 | 1 | B - A | B minus A (or A' + B + 1) |
| M | S1 | S0 | C0 | FUNCTION | OPERATION (bit wise) |
| 1 | 0 | 0 | X | AiBi | AND |
| 1 | 0 | 1 | X | Ai + Bi | OR |
| 1 | 1 | 0 | X | Ai Bi | XOR |
| 1 | 1 | 1 | X | (Ai Bi)’ | XNOR |

3. ผลการทดลอง

4. สรุปผลการทดลอง

5. คำถามหลังการทดลอง

- การออกแบบวงจรบวกมีกี่วิธีอะไรบ่าง?

- บอกหลักการทำวงจรบวกให้เป็นวงจรลบ

- ถ้าต้องการออกแบบวงจรบวกและลบเลขขนาด 128 บิทจะต้องออกแบบอย่างไร

Test.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity test is

Port (

M, S1, S0 : in  STD_LOGIC;

Cin_in : in  STD_LOGIC;

A4, A3, A2, A1, A0 : in STD_LOGIC;

B4, B3, B2, B1, B0 : in STD_LOGIC;

Cin, GB, FB, GA, FA : out STD_LOGIC;

Mini0, Mini1, Mini2, Mini3, Mini4 : out STD_LOGIC;

C0, C1, C2, C3, C4 : out STD_LOGIC;

F0, F1, F2, F3, F4 : out STD_LOGIC;

Co : out STD_LOGIC

);

end test;

architecture Behavioral of test is

signal SEL : STD_LOGIC_VECTOR(1 downto 0);

signal Cin_s : STD_LOGIC;

signal GB_s  : STD_LOGIC;

signal FB_s  : STD_LOGIC;

signal GA_s  : STD_LOGIC;

signal FA_s  : STD_LOGIC;

signal X0, X1, X2, X3, X4 : STD_LOGIC;

signal Y0, Y1, Y2, Y3, Y4 : STD_LOGIC;

signal Carry0, Carry1, Carry2, Carry3, Carry4 : STD_LOGIC;

signal M0, M1, M2, M3, M4 : STD_LOGIC;

begin

-- Control Signals

SEL <= S1 & S0;

Cin_s <= (not M) and Cin_in;

GB_s  <= M or S1 or S0;

FB_s  <= (not M) and S1 and (not S0);

GA_s  <= '1';

FA_s  <= (not M) and S1 and S0;

Cin <= Cin_s;

GB  <= GB_s;

FB  <= FB_s;

GA  <= GA_s;

FA  <= FA_s;

-- Mini0

with SEL select

M0 <= (A0 and B0)  when "00",

(A0 or B0)   when "01",

(A0 xor B0)  when "10",

(A0 xnor B0) when others;

-- Mini1

with SEL select

M1 <= (A1 and B1)  when "00",

(A1 or B1)   when "01",

(A1 xor B1)  when "10",

(A1 xnor B1) when others;

-- Mini2

with SEL select

M2 <= (A2 and B2)  when "00",

(A2 or B2)   when "01",

(A2 xor B2)  when "10",

(A2 xnor B2) when others;

-- Mini3

with SEL select

M3 <= (A3 and B3)  when "00",

(A3 or B3)   when "01",

(A3 xor B3)  when "10",

(A3 xnor B3) when others;

-- Mini4

with SEL select

M4 <= (A4 and B4)  when "00",

(A4 or B4)   when "01",

(A4 xor B4)  when "10",

(A4 xnor B4) when others;

Mini0 <= M0;

Mini1 <= M1;

Mini2 <= M2;

Mini3 <= M3;

Mini4 <= M4;

-- Arithmetic Generator

X0 <= FA_s xor (GA_s and A0);

Y0 <= FB_s xor (GB_s and B0);

X1 <= FA_s xor (GA_s and A1);

Y1 <= FB_s xor (GB_s and B1);

X2 <= FA_s xor (GA_s and A2);

Y2 <= FB_s xor (GB_s and B2);

X3 <= FA_s xor (GA_s and A3);

Y3 <= FB_s xor (GB_s and B3);

X4 <= FA_s xor (GA_s and A4);

Y4 <= FB_s xor (GB_s and B4);

-- Bit 0

F0 <= (Cin_s xor X0 xor Y0)

when M='0'

else M0;

Carry0 <= (Cin_s and (X0 or Y0))

or

(X0 and Y0);

C0 <= Carry0;

-- Bit 1

F1 <= (Carry0 xor X1 xor Y1)

when M='0'

else M1;

Carry1 <= (Carry0 and (X1 or Y1))

or

(X1 and Y1);

C1 <= Carry1;

-- Bit 2

F2 <= (Carry1 xor X2 xor Y2)

when M='0'

else M2;

Carry2 <= (Carry1 and (X2 or Y2))

or

(X2 and Y2);

C2 <= Carry2;

-- Bit 3

F3 <= (Carry2 xor X3 xor Y3)

when M='0'

else M3;

Carry3 <= (Carry2 and (X3 or Y3))

or

(X3 and Y3);

C3 <= Carry3;

-- Bit 4

F4 <= (Carry3 xor X4 xor Y4)

when M='0'

else M4;

Carry4 <= (Carry3 and (X4 or Y4))

or

(X4 and Y4);

C4 <= Carry4;

-- Final Carry

Co <= Carry4;

end Behavioral;

Tb_test.vhdl

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_test is

end tb_test;

architecture behavior of tb_test is

component test

Port (

M, S1, S0 : in  STD_LOGIC;

Cin_in : in  STD_LOGIC;

A4, A3, A2, A1, A0 : in STD_LOGIC;

B4, B3, B2, B1, B0 : in STD_LOGIC;

Cin, GB, FB, GA, FA : out STD_LOGIC;

Mini0, Mini1, Mini2, Mini3, Mini4 : out STD_LOGIC;

C0, C1, C2, C3, C4 : out STD_LOGIC;

F0, F1, F2, F3, F4 : out STD_LOGIC;

Co : out STD_LOGIC

);

end component;

signal M,S1,S0,Cin_in : STD_LOGIC;

signal A4,A3,A2,A1,A0 : STD_LOGIC;

signal B4,B3,B2,B1,B0 : STD_LOGIC;

signal Cin,GB,FB,GA,FA : STD_LOGIC;

signal Mini0,Mini1,Mini2,Mini3,Mini4 : STD_LOGIC;

signal C0,C1,C2,C3,C4 : STD_LOGIC;

signal F0,F1,F2,F3,F4 : STD_LOGIC;

signal Co : STD_LOGIC;

begin

DUT : test

port map(

M => M,

S1 => S1,

S0 => S0,

Cin_in => Cin_in,

A4 => A4,

A3 => A3,

A2 => A2,

A1 => A1,

A0 => A0,

B4 => B4,

B3 => B3,

B2 => B2,

B1 => B1,

B0 => B0,

Cin => Cin,

GB => GB,

FB => FB,

GA => GA,

FA => FA,

Mini0 => Mini0,

Mini1 => Mini1,

Mini2 => Mini2,

Mini3 => Mini3,

Mini4 => Mini4,

C0 => C0,

C1 => C1,

C2 => C2,

C3 => C3,

C4 => C4,

F0 => F0,

F1 => F1,

F2 => F2,

F3 => F3,

F4 => F4,

Co => Co

);

process

begin

-- A = 10101 1+4+16 = 21

-- B = 01100 4+8 = 12

A4 <= '1';

A3 <= '0';

A2 <= '1';

A1 <= '0';

A0 <= '1';

B4 <= '0';

B3 <= '1';

B2 <= '1';

B1 <= '0';

B0 <= '0';

-- A

M <= '0'; S1 <= '0'; S0 <= '0'; Cin_in <= '0';

wait for 20 ns;

-- A+1

M <= '0'; S1 <= '0'; S0 <= '0'; Cin_in <= '1';

wait for 20 ns;

-- A+B

M <= '0'; S1 <= '0'; S0 <= '1'; Cin_in <= '0';

wait for 20 ns;

-- A+B+1

M <= '0'; S1 <= '0'; S0 <= '1'; Cin_in <= '1';

wait for 20 ns;

-- A+B'

M <= '0'; S1 <= '1'; S0 <= '0'; Cin_in <= '0';

wait for 20 ns;

-- A-B

M <= '0'; S1 <= '1'; S0 <= '0'; Cin_in <= '1';

wait for 20 ns;

-- A'+B

M <= '0'; S1 <= '1'; S0 <= '1'; Cin_in <= '0';

wait for 20 ns;

-- B-A

M <= '0'; S1 <= '1'; S0 <= '1'; Cin_in <= '1';

wait for 20 ns;

-- AND

M <= '1'; S1 <= '0'; S0 <= '0'; Cin_in <= '0';

wait for 20 ns;

-- OR

M <= '1'; S1 <= '0'; S0 <= '1'; Cin_in <= '0';

wait for 20 ns;

-- XOR

M <= '1'; S1 <= '1'; S0 <= '0'; Cin_in <= '0';

wait for 20 ns;

-- XNOR

M <= '1'; S1 <= '1'; S0 <= '1'; Cin_in <= '0';

wait for 20 ns;

wait;

end process;

end behavior;